GlobalSat ND-100 GPS F/W Update Document

Note:

Before updating, please confirm the power of PC/NB is full enough to avoid PC/NB down while updating. Also, please do not operate any other programs in updating process; otherwise, it may cause update failure or device damage.

Please install ND-100 USB driver before this F/W update. If not, please get ND-100 USB Driver from enclosed CD or download and install it from GlobalSat website. (http://www.globalsat.com.tw/index.htm)

Update Processing Steps:

- Please insert ND-100 to PC/NB USB port and it will connect automatically.

- After the connection is ready, please get the specific COM port between ND-100 and PC/NB, the steps as below；

  - Please use your mouse and move the cursor to “My Computer” icon and click the right key and select “Properties”.

  - Under “Properties” window, please select “Hardware” subpage and click the “Device Manager” icon..

  - On Device Manager page, please click the “Ports”  item and you can get the COM port of ND-100.

(Please refer to below screenshots as reference of the steps)

4. After confirming ND-100 COM port,  please execute (double click) “AutoFWUpdate_ND100.exe” which you downloaded in Step 1 and start to update the F/W of ND-100.

5.Please select the corresponding COM port of ND-100 (Refer to Step 3.) and specific baud rate(38400) for parameter setting as the followings；

Please select the corresponding comport from step 3.

Please select 4800

6. After completing the settings, please click “OK” to proceed the F/W update.

7. The update screenshot is as below. After the update completes, the update program will turn off automatically. (Please do not remove ND-100 from PC/NB or operate while updating.)

- After the updating process runs out, ND-100 update is finished. You can follow the product manual to use MiniCDU.exe for verifying GPS fix.